Department of Environmental Health & Safety

Occupational Health & Safety Office

321 Ryan Street,

Essex, Vermont 05452

SCAFFOLD INSPECTION CHECKLIST

- Include and submit checklist(s) to UVM Project Supervisor(s) and Occupational Health and Safety Office ohso@uvm.edu.

- To be completed by a competent person.

- If repairs are needed, IMMEDIATELY pull out of service, label “Do Not Use”, and notify your supervisor.

| Project Number |  | Description |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Click here to enter text. |  | Click here to enter text. |  |  |  |  |
| Department |  | Building |  |  | Location |  |
| Click here to enter text. |  | Click here to enter text. |  |  | Click here to enter text. |  |
| Inspection Completed By (Signature): |  |  |  |  |  | Date: |
| Click here to enter text. Print Name |  | Click here to enter text. Email |  |  |  | Click here to enter text. Phone |
| Scaffold Type (select all that apply) |  |  |  |  |  |  |
| ☐ Job-Built | ☐ Temporary |  | ☐ Manufactured | ☐ Other (specify): Click here to enter text. |  |  |
| Scaffold Material (select all that apply) |  |  |  |  |  |  |
| ☐ Wood | ☐ Metal |  | ☐ Rope/Wire | ☐Other (specify): Click here to enter text. |  |  |

| Scaffold Inspection Checklist |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Item # | Item | YES | NO | N/A | NEEDS REPAIR |
| 1 | Scaffolds were designed by a licensed professional engineer competent in scaffolding | ☐ | ☐ | ☐ | ☐ |
| 2 | Scaffolds were erected under the supervision of a trained and competent person | ☐ | ☐ | ☐ | ☐ |
| 3 | Scaffolds are in good repair and inspected by a competent person prior to use | ☐ | ☐ | ☐ | ☐ |
| 4 | Planking is made of 2 x 10-inch scaffold grade lumber or metal | ☐ | ☐ | ☐ | ☐ |
| 5 | Planking spans no more than 10 feet for light trades (25 pounds per square foot, psf), 8 feet for medium trades (50 psf) or 6 feet for heavy trades (75 psf) | ☐ | ☐ | ☐ | ☐ |
| 6 | Planks overhang supports by 6 (minimum) to 12 inches (maximum) | ☐ | ☐ | ☐ | ☐ |
| 7 | Uprights are plumb (vertical) and securely braced to prevent swaying | ☐ | ☐ | ☐ | ☐ |
| 8 | The scaffold is tied off and secured to a stable structure | ☐ | ☐ | ☐ | ☐ |
| 9 | All open sides above 4 feet have 42-inch high guardrails with a 21-ich midrail | ☐ | ☐ | ☐ | ☐ |
| 10 | Guardrail supports are no more than 8 feet apart | ☐ | ☐ | ☐ | ☐ |
| 11 | All open sides above 10 feet have a 4-inch high toe-board | ☐ | ☐ | ☐ | ☐ |
| 12 | Ladders for access extend 3 feet above the platform and are securely attached | ☐ | ☐ | ☐ | ☐ |
| 13 | No work is performed during windy conditions | ☐ | ☐ | ☐ | ☐ |
| 14 | Other (specify): Click here to enter text. | ☐ | ☐ | ☐ | ☐ |
| Corrective Actions Taken |  | ☐ | ☐ | ☐ |  |
| #Enter here | Describe: Click here to enter text. |  |  | Date: Click here to enter text. |  |
| #Enter here | Describe: Click here to enter text. |  |  | Date: Click here to enter text. |  |
| #Enter here | Describe: Click here to enter text. |  |  | Date: Click here to enter text. |  |
| #Enter here | Describe: Click here to enter text. |  |  | Date: Click here to enter text. |  |
| #Enter here | Describe: Click here to enter text. |  |  | Date: Click here to enter text. |  |
| Other Comments: Click here to enter text. |  |  |  |  |  |