National Register Photo 

Policy Factsheet

Selecting a Digital Camera

BEST:
Six megapixel or greater digital SLR camera

Acceptable:  Two – five megapixel point-and-shoot digital camera


Not acceptable:  Camera phones, disposable or single-use digital cameras, digital cameras with fewer than two megapixels of resolution
Taking the Picture
· Image file format (Set the camera for highest image quality).
BEST: Tag Image File format (TIFF) or RAW format images.  This allows for the best image resolution. 

Acceptable:  JPEGs converted to TIFFs, by a computer conversion process, are acceptable; however, JPEGs must not be altered in any way prior to conversion, (other than renaming them).  

Do not use the JPEG setting on the camera, if a higher quality setting is available. 

RGB color digital TIFFs are preferred.

· Digital Camera Resolution (Set the camera to the maximum or largest pixel dimension the camera allows).
BEST:
Six megapixels or greater (2000 x 3000 pixel image) at 300 dpi
Acceptable:  Minimum two megapixels (1200 x 1600 pixel image) at 300 dpi

Renaming the digital TIFF image 

All digital image files must be renamed using a standard naming format.

Burning the Images onto an Archival Disk

A CD or DVD containing all TIFF images must accompany the photos.  

Reminder:  JPEGs converted to TIFFs, by a computer conversion process, are acceptable; however, JPEGs must not be altered in any way prior to conversion, (other than renaming them).  When image is open on your computer, right click and you will see the image properties (Dimensions, dpi, etc.).
Best:  CD-R Archival Gold or DVD-R Archival Gold disk  

Acceptable:  CD-R, DVD-R, or any disk obtained from a commercial photo processor. 

Not acceptable:  CD-RW or DVD-RW (if packaging says “rewriteable” do not use).
Labeling the Disk

Best:  Labels printed directly on the disk by laser printer (non-adhesive).
Acceptable:  Hand-written labels using CD/DVD safe markers OR other markers (Sharpies) 

Not Acceptable:
Ammonia/solvent-based markers or adhesive stickers


Printing the Images 

Print photos at 300 dpi (select this option in your computer’s print menu).
Selecting the Paper and Inks
· Archival quality paper:

BEST:
Manufacturer recommended paper for photograph prints
Some examples:
· Epson Premium Glossy Paper

· Kodak Ultra Photo Premium

· HP Professional Satin Photo Paper

· Matte Epson Ultra Premium Glossy Photo Paper

· HP Premium Plus Photo Paper

Acceptable:  Commercially printed color prints are acceptable (if accompanied by a disk containing the image files produced at the time the prints were made).

Not acceptable:  Regular copy/printer papers or the disk only, without prints

· Printer Inks:
 
BEST:
Manufacturer recommended ink for photograph printing

Some examples:
-
Epson UltraChrome K3
· Kodak No. 10 Pigmented Inks

· HP Vivera Pigment Inks

· Epson Claria “Hi-Definition Inks”

· Epson DuraBrite Ultra Pigmented Inks

· HP Vivera 95 dye-based inks
Note:  NPS does not endorse any particular commercial product or process.  The product examples indicated have met established archival standards but do not represent a comprehensive list.  Any questions concerning other possible products should be directed to Alexis Abernathy at (202) 354-2236 or e-mail: alexis_abernathy@contractor.nps.gov.

Identifying Photographic Prints

Each photograph must be numbered and that number must correspond to the photograph number on the photo log.  For simplicity, the name of the photographer, photo date, etc. may be listed once on the photograph log and doesn’t need to be labeled on every photograph. 


Best:  Write the label information within the white margin on the front of the photograph using an archival photo labeling pen.  Label information can also be generated by computer and printed directly in the white margin (no adhesive labels).  

Acceptable:   If information is placed on the back of the photograph, write the information using a soft lead pencil or archival photo-labeling pen. 

Do not print information on the actual image – use only the photo margin or back of the photograph for labeling.

At a minimum, photographic labels must include the following information:  

Photograph number, Name of the Property, County, and State.  

Labeling the photographs

NR Nomination Photograph Log Page

Examples of acceptable photo pages
Name of Property:


Belcher-Nixon Building

City or Vicinity:



Ensley (Birmingham)

County:




Jefferson County

State:




AL

Name of Photographer:


David B. Schneider

Date of Photographs:


August  2008

Location of Original Digital Files:

411 E. 6th St., Anniston, AL 36207

Photo #1 (AL_JeffersonCounty_BelcherNixonBld_0001)

South façade (left) and east elevation (right), camera facing northwest.

OR

Name of Property:


Belcher-Nixon Building

City or Vicinity:



Ensley (Birmingham)

County:




Jefferson County

State:




AL

Name of Photographer:


David B. Schneider

Date of Photographs:


August  2008

Location of Original Digital Files:

411 E. 6th St., Anniston, AL 36207

Number of Photographs:  


10

Photo #1 

South façade (left) and east elevation (right), camera facing northwest.

OR

Name of Property:


Belcher-Nixon Building

City or Vicinity:



Ensley (Birmingham)

County:




Jefferson County

State:




AL

Name of Photographer:


David B. Schneider

Date of Photographs:


August  2008

Location of Original Digital Files:

411 E. 6th St., Anniston, AL 36207

AL_JeffersonCounty_BelcherNixonBld_0001

South façade (left) and east elevation (right), camera facing northwest.
35 mm Photography 
Use the following standards:
Selecting a 35mm Camera

BEST:


35MM SLR Camera
Acceptable:

35MM point-and-shoot camera

Not acceptable:

Disposable Cameras

Selecting the Film

Acceptable: 

35MM black/white film 

Or

         



35MM color film with accompanying disk containing the image files

Choosing Photographic Paper

Acceptable:

Photographic paper specifically designed for black/white

prints 

Or

35mm black/white images printed on paper designed for color prints with an accompanying disk containing digital copies of the images (Disk generated at the time of developing the film) 

Or
35mm color images printed on paper designed for color images with an accompanying disk containing digital copies of the images
Labeling the Disk & Naming the Files


Follow the same disk labeling and renaming the digital file processes as outlined under the digital photography policy guidelines.

If you use a commercial photo printer and receive a disk of image files to submit with your nomination, you will not be required to rename the files.  That disk may be submitted as received from your photo processor.

Use of National Register Photographs


By allowing a photograph to be submitted as official documentation, photographers grant permission to the National Park Service to use the photograph for print and electronic publication, and for other purposes, including but not limited to, duplication, display, distribution, study, publicity, and audiovisual presentations.

Embedding Images

Color and black-and-white images may not be embedded within the text of a nomination.

As of March 1, 2010, any nominations with embedded images within the text will not be accepted and will be returned for correction.  
Historic photographs, views, or maps are acceptable--on separate Continuation Sheets as Additional Documentation--placed at the end of a nomination and not embedded within the text. These items can be labeled as figures (e.g. Fig. 1, Fig 2) and referenced by this label within the nomination text (e.g. See Figure 1). 
An “Index of Figures” (similar to a photograph log) identifying these figures, should also be included in the Additional Documentation section.
Guidelines for Photographic Coverage


Photographs submitted to the National Register of Historic Places and the National Historic Landmarks Survey as official documentation should be clear, well-composed, and provide an accurate visual representation of the property and its significant features. They must illustrate the qualities discussed in the description and statement of significance. Photographs should show historically significant features and also any alterations that have affected the property’s historic integrity.

The necessary number of photographic views depends on the size and complexity of the property. Submit as many photographs as needed to depict the current condition and significant features of the property. A few photographs may be sufficient to document a single building or object. Larger, more complex properties and historic districts will require a number of photos. Prints of historic photographs may supplement documentation and be particularly useful in illustrating changes that have occurred over time. 

Buildings, structures, and objects:

Submit photographs showing the principal facades and the setting in which the property is located. 

Additions, alterations, intrusions, and dependencies should appear in the photographs. 

Include views of interiors, outbuildings, landscaping, or unusual features if they contribute to the significance of the property. 

Historic and archeological sites:

Submit photographs showing the condition of the site and any above-ground or surface features and disturbances. 

If relevant to the evaluation of significance, include drawings or photographs illustrating artifacts that have been removed from the site. 

At least one photograph must show the physical environment and topography of the site. 

Architectural and Historic Districts (key all photographs to the sketch map for the district):
Submit photographs showing major building types and styles, pivotal buildings and structures, and representative noncontributing resources. 

Streetscapes and landscapes are recommended. Aerial views may also be useful. Views of significant topographic features and spatial elements should also be submitted. 

Views of individual buildings are not necessary if streetscape views clearly illustrate the significant historical and architectural qualities of the district. 

Archeological Districts:

Submit photographs of the principal sites and site types within the district following the guidelines for archaeological sites (see above). 

Questions?
Please contact Alexis Abernathy at (202) 354-2236 or e-mail: alexis_abernathy@contractor.nps.gov. 
The TIFF file name must include:


State_county_property name (or district name)_0001


(Use zeros in image numbers to create 4 digit number, e.g. 0002, 0003, etc.)





Example for individual properties:


AL_Jefferson County_Elizabeth Brown House_0001





Example for district labels:


AL_Jefferson County_Birmingham Commercial Historic District_0125





Example for nominations within MPS:


AL_Jefferson County_NorwoodMPS_EBrownHouse_0001








Elizabeth Brown House


Birmingham, Jefferson Co.


AL




















National Register


Nomination – June 2009





Acceptable Examples:


AL_Jefferson County_Birmingham Commercial Historic District_0001





OR





Photograph 1 of 25:   AL_Jefferson County_Birmingham Commercial Historic District_0001





OR





Birmingham Commercial Historic District, Jefferson County, AL


1 of 25





OR





Birmingham Commercial Historic District


Jefferson Co., AL


Photo 1 of 25




















8

