Why It’s 1:00 am and I’m Not Sleeping by Matt DeWolfe

For anyone who gives a hoot, a better explanation (I think) as to why Kondolf’s criticism of Leopold’s proposed modification to the pebble count is without basis:

First, let’s review Kondolf’s criticism:

“Leopold proposed that pebble count analysis be adjusted to compensate for a ‘bias… towards larger sizes which, because of their area are more likely to be picked up.” However, as long as grain volume is proportional to grain weight (true with constant density), there is no bias in a random pebble count. If larger grains are more likely to be encountered, it is because they occupy a greater part of the cross sectional area of the slice (the surface) and thus a greater part of the volume of the three dimensional deposit. Thus, there is no theoretical justification for decreasing the actual percentages observed for larger stones so long as the pebble count is conducted correctly (and the count is truly blind and thus random).”

Leopold’s justifications in his own words:

“For the geomorphologists working with coarse material, the sieve method is often impossible to use because the sample of required size cannot be carried to the laboratory or even sieved in the field. It is necessary then to use a method of counting the number of surface pebbles of various sizes, the sample being made up of a random selection of individual rocks within an area of several hundreds of square feet (Wolman, 1954). Though this method or variants of it have been used for field studies, it suffers from the important disadvantage of giving results not comparable with the usual sieved analysis. The counting method gives a size distribution of number of rocks, not their weights.” (emphasis added).

“The field counting method is biased toward the large sizes. Because the method consists of picking up a rock just touched by the finger, eyes averted or closed, the larger the rock the greater the probability of being touched relative to neighboring smaller individuals… The probability of the operator touching a given rock increases with the exposed area of rock. Therefore the large rocks tend to be picked up too frequently relative to their numeric frequency.”

The key here is that what the Pebble Count method is supposed to tell us is the % of pebbles of certain class based on number only. From this, the weight distributions are typically estimated as discussed below. What is important to the pebble count is that the sample is completely random (as Kondolf agrees), therefore each pebble must be afforded the exact same chance of being selected regardless of size. What Leopold argues is that an increase in size distorts this chance, increasing the odds of one pebble over another being selected. To correct for this he suggests:

“The increased probability [of a large pebble being selected at random]  is proportional to the projected area or therefore to the square of the mean diameter. The number of rocks in each size class should therefore be weighted by a factor inversely proportionate to the square of the diameter of the B axis.”

Remember that this is the Pebble Count procedure. The eventual aim of the procedure is to simulate the bulk sample sieve analysis in the field, but this is achieved through determining a % of count for each class first. The bulk sieve analysis seeks to determine at what percentage (by weight) each size class is represented in the sample. This is done by taking the weight of a sample that stops on a given sieve and dividing by the overall sample weight. However, in the Pebble Count method, the % based on weight is determined based upon the % by number:

(# counted in class)* (Avg weight per pebble in class) = Total Weight in Class

% by Weight in Class = (Total Weight in Class)/(Overall Total Weight of all pebbles)

(Note the standard weights for pebbles in each class are given in Leopold’s paper)

For example, here are Kondolf’s figures, omitting the <8mm size class.

Table 1. Kondolf’s Method (no adjustment for diameter)

| Size Class | No. Rocks Sampled (n) | Percent of total # rocks sampled | Weight (w) of rock in class in grams | Total weight sampled (n * w) in grams | % of overall weight = (n*w)/Total w |
| --- | --- | --- | --- | --- | --- |
| 256 | 2 | 1.7 | 40000 | 66115.7 | 25.054 |
| 180 | 4 | 3.3 | 15000 | 49586.8 | 18.791 |
| 128 | 16 | 13.2 | 5600 | 74049.6 | 28.061 |
| 90 | 30 | 24.8 | 2100 | 52066.1 | 19.730 |
| 64 | 30 | 24.8 | 700 | 17355.4 | 6.577 |
| 45 | 19 | 15.7 | 255 | 4004.1 | 1.517 |
| 32 | 6 | 5.0 | 94 | 466.1 | 0.177 |
| 22.6 | 7 | 5.8 | 34 | 196.7 | 0.075 |
| 16 | 4 | 3.3 | 12 | 39.7 | 0.015 |
| 11.3 | 2 | 1.7 | 4.5 | 7.4 | 0.003 |
| 8 | 1 | 0.8 | 1.6 | 1.3 | 0.001 |
| TOTAL | 121 | 100.0 |  | 267445.7. | 100.0 |

Next, here are the results using Leopolds adjustment for class size:

Table 2. Results adjusting for class size.

| Size Class | No. Rocks Sampled (n) | Percent of total # rocks sampled | Weight (w) of rock in class in grams | Total weight sampled (n * w) in grams | Total Weight / d2 (Leopold method) | % of overall weight = (n*w)/ (Total w *d2) |
| --- | --- | --- | --- | --- | --- | --- |
| 256 | 2 | 1.7 | 40000 | 66115.7 | 1.008846 | 4.86 |
| 180 | 4 | 3.3 | 15000 | 49586.8 | 1.530456 | 7.37 |
| 128 | 16 | 13.2 | 5600 | 74049.6 | 4.519628 | 21.75 |
| 90 | 30 | 24.8 | 2100 | 52066.1 | 6.427916 | 30.94 |
| 64 | 30 | 24.8 | 700 | 17355.4 | 4.237151 | 20.39 |
| 45 | 19 | 15.7 | 255 | 4004.1 | 1.977349 | 9.52 |
| 32 | 6 | 5.0 | 94 | 466.1 | 0.455191 | 2.19 |
| 22.6 | 7 | 5.8 | 34 | 196.7 | 0.385101 | 1.85 |
| 16 | 4 | 3.3 | 12 | 39.7 | 0.154959 | 0.75 |
| 11.3 | 2 | 1.7 | 4.5 | 7.4 | 0.058251 | 0.28 |
| 8 | 1 | 0.8 | 1.6 | 1.3 | 0.020661 | 0.10 |
| TOTAL | 121 | 100.0 |  | 267445.7. |  | 100 |

As can clearly be seen the results are strikingly different for percent of total sample weight, with a much greater value being given for the upper size classes. The question that needs to be addressed is whether or not the total weight in each class is accurate by adjusting or not adjusting (or neither).

Now for the fun part: (see last page for diagram)

Assume we have the following population (ie the actual proportion of pebbles – this is clearly hypothetical to make a point, but any known population could be used to do this test):

6 pebbles * 30mm class (it is assumed that each pebble in each class is spherical)

8 pebbles * 20mm class

14 pebbles * 10mm class

We are sampling a grid 100mm*100mm, perfectly square, and our sample, conveniently, fits neatly within this grid. We have 100 samples to take so each sample plot in our grid is 10mm*10mm (100mm2).

| Pebble Size | Surface area taken up per pebble | Actual Number | Total Area Actually taken up |
| --- | --- | --- | --- |
| 30 mm | 302 = 900 mm2 = 9 sample plots large | 6 | 6* 900 = 5400 mm2 = 54 sample plots |
| 20mm | 202 = 400 mm2 = 4 sample plots large | 8 | 8*400 = 3200 mm2 = 32 sample plots |
| 10 mm | 102 = 100 mm2 = 1 sample plot | 14 | 14*100 = 1400 mm2 = 14 sample plots |

Each sample plot is sampled once for a total of 100 samples. The results are:

| Pebble Size | Pebble Count |
| --- | --- |
| 30 mm | 54 |
| 20mm | 32 |
| 10 mm | 14 |

Total			100

Recall that our goal is to mimic the bulk sieve analysis by determining the % weight in for each size category. Let us cast away the assumption that weight is directly proportional to area. Using the weights by classes given above by Leopold I have determined a relationship between class (based upon B axis measurement) and weight:

Using this regression equation the weight for standard pebbles in our hypothetical situation are:

| Pebble Size | Pebble Weight |
| --- | --- |
| 30 mm | 77.6 g |
| 20mm | 23.6 g |
| 10 mm | 3.1 g |

Now, the actual population consists of six 30mm pebbles, eight 20mm pebbles, and fourteen 10mm pebbles. The actual % weight for each class then is:

| Pebble Size | Pebble Weight | Actual Pebble # | Total Actual Weight | % Actual Weight per class |
| --- | --- | --- | --- | --- |
| 30 mm | 77.6 g | 6 | 465.6 g | 66.7 |
| 20mm | 23.6 g | 8 | 188.8 g | 27.0 |
| 10 mm | 3.1 g | 14 | 43.26 g | 6.2 |

Total		697.66		~100

This is the actual weight distribution that would be determined from sieve analysis using our hypothetical situation. Now compare to the pebble count result:

Assuming that 100 pebbles are taken across the grid at evenly spaced points then the counts would be:

| Pebble Size | Pebble Count |
| --- | --- |
| 30 mm | 54 |
| 20mm | 32 |
| 10 mm | 14 |

Again, using the same weights for each class:			          Compare to

| Pebble Size | Pebble Weight | Pebble Count | Total weight | % Weight per class | Actual % weight per class |
| --- | --- | --- | --- | --- | --- |
| 30 mm | 77.6 g | 54 | 4192g | 84.0 | 66.7 |
| 20mm | 23.6 g | 32 | 756 g | 15.15 | 27.0 |
| 10 mm | 3.1 g | 14 | 43.3 g | 0.87 | 6.2 |

Clearly the two are not exactly the same, the much large stones are over represented and, if this line of thinking is correct, then Kondolf’s assertion that “… there is no theoretical justification for decreasing the actual percentages observed for larger stones .” appears incorrect. Now let’s consider Leopold’s modification:

His modification is to divide the total weight in each class by the square of the diameter for that class, and then to determine the % of the adjusted weight as the final value:

|  |  |  | Adjusted Weight |  |  |
| --- | --- | --- | --- | --- | --- |
| Pebble Size | D2 | Total weight | Total weight/D2 | % Adjusted weight per class | Actual % weight per class |
| 30 mm | 900 mm2 | 4192g | 4.65 | 66.72 | 66.7 |
| 20mm | 400 mm2 | 756 g | 1.89 | 27.07 | 27.0 |
| 10 mm | 100 mm2 | 43.3 g | 0.432 | 6.20 | 6.2 |

Total	      6.98

These results match perfectly!

Question: This is an unreal setup, would this work in a real field situation?

First of all, the arrangement of the pebbles in the diagram below clearly isn’t important, any way they’re arranged if sampled properly will give the same field data (100 samples each taken in an even grid).

Take this example and blow it up 100 times, so that each of the grids I created above was multiplied 100 times. Then sample each of these randomly once. If it’s a true random sample then you would expect to get the same general proportions of samples. Increasing the sample size infinitely would give the exact population. So you’d always collect 54% 30mm pebbles, 32% 20 mm pebbles, and 14% 10mm pebbles. The above is an idealized situation where the sample exactly represents the population. There is no other way to show this theoretically that I can come up with. The point is that the increased stone size does bias the result and that Leopolds argument corrects for this bias. I would gladly entertain any discussion as I’m still going over this, but it seems to me that this works.

20 mm									10 mm

30 mm

Total Grid: 100mm * 100mm with 100 sample plots 10mm*10 mm